FLUG Wireless Mesh Network Talk

By Jeff Liebermann     jeffl@cruzio.com

2015-02-28

This document located at:

http://802.11junk.com/jeffl/FLUG-talk-2015-02-28/

- How Wi-Fi works

  - One transmitter on the air at a time.

  - CSMA/CA.  Half Duplex.  Store and forward repeaters.

  - Half or full duplex backhaul.

  - Indoor and outdoor are very different.

- Buzzwords etc.

  - Mesh is ad-hoc networking.  All wi-fi is on MAC layer 2.

  - Repeaters and WDS.  Partial mesh, no routing.  Acts as both access point and store and forward repeater at same time.

  - Metricom.  Partial mesh with geographic routing.

  - WDS, NPG, OLSR, etc

- Routing

  - Each hop reduces max speed by using air time to send extra packets.

  - Table OLSR – Optimized link state routing.

  - Dynamic OLSR – self organizing.

  - Hierarchical CBRP (Cluster based routing protocol)

  - AODV – On Demand Distance Vector.

  - CBRD – Cluster Based Routing Protocol.

- Types of mesh networks.

  - Ad-hoc pretzel.  No controller as in infrastructure.

  - Single band store and forward.  Most common.

  - Dual band.  One for users, one for backhaul.

- Advantages of Wireless Mesh Networks.

  - Saves on cost of wired backhaul.

  - Self configuring, self provisioning, self organizing, self healing, self-monitoring.

  - Grow or move as needed.

  - Standardized as 802.11s with HWMP routing.  Used by OLPC for small mesh networks.  http://en.wikipedia.org/wiki/IEEE_802.11s

- Disadvantages or why mesh sucks.

  - Doesn’t scale.

  - Maximum speed limited my number of hops.

  - Congestion near wired access points.

  - High collision rate due to hidden nodes.

  - Usually requires omnidirectional antennas, which are not optimum pattern for rooftops.  Directional antennas create hidden nodes.

  - What’s wrong with this picture?  http://wndw.net/pdf/wndw3-en/ch08-mesh-networking.pdf

  - 802.11 not designed for large networks.  Polling is more efficient for WISP.  http://www.solectek.com/files/pdf/techtalk/White_Paper-polling_MAC_advantage_v1.2.pdf

  - Indoor vs Outloor.  Few AP’s versus thousands.

  - Low percentage of packet delivery due to interference, hidden nodes, and retransmissions.  MIT Rooftop Networks.

  - http://802.11junk.com/jeffl/FLUG-talk-2015-02-28/MIT-roofnet-b.pdf

  - http://pdos.csail.mit.edu/grid/pubs.html

  - http://pdos.csail.mit.edu/grid/

  - Speed drops to the slowest speed (802.11b=1Mbit/sec, 802.11a/g=6Mbits/sec)

  - Limited encryption.  Must use RADIUS server.  Cannot encrypt IP header.

  - http://telehash.org

  - Too many wireless mesh routing protocols (70)  http://en.wikipedia.org/wiki/Wireless_mesh_network#Routing_protocols

  - Google does not support Ad-Hoc networking in Android.

- Examples

  - San Francisco (6 -> 22 nodes per square mile)

  - Philadelphia  (Cost over-runs)

  - Meraki.  Bought by Cisco for $1.6 billion in 2012 mostly for cloud management technology.

  - Tropos

  - Metricom

- Links and reading

  - Wireless Networking in the Developing World  http://wndw.net

  - http://www.muniwireless.com

  - https://www.eero.com

- Speed slowdown with repeater demo

  - http://www.techrepublic.com/blog/linux-and-open-source/using-jperf-to-check-network-performance/

  - Iperf  https://iperf.fr/

  - Jperf  http://sourceforge.net/projects/iperf/files/

  - Jperf  https://code.google.com/p/xjperf/downloads/detail?name=jperf-2.0.2.zip