FLUG Talk on SSD for Linux and KODI

By Jeff Liebermann

2015-10-24

Why buy an SSD?

- Speed, reliability, low power, shock proof, small, prices dropping.

- Samsung 850 EVO.  120GB=$64, 250GB=$85, 500GB=$164, 1TB=$340, 2TB=$710.

- Types:  2.5”, M.2, mSATA

- Differences from USB flash drive.  High end SD cards (SDXC) have wear leveling.

PC Requirements

- SATA 2 or 3

- TRIM in BIOS and OS (or in software as in older OS/X)

- Modern OS with TRIM support.

- 2.5” SATA slot or socket.

- AHCI in BIOS (not IDE emulation)

How it works

- Read approx 540 Mbytes/sec sequential or 100K I/O ops with 4 Kbyte blocks.

- Write is about 50 to 100% of read speed depending on technology.

- Block erase method.  SSD cannot overwrite data.  Must erase first.  (Read 256KB, erase, modify, write back).

- Wear Leveling.  Writes can shorten SSD lifetime.  Wear Leveling distributes writes.

- TRIM (garbage collection).  Reduces Write Amplification.

https://en.wikipedia.org/wiki/Write_amplification

- DRAM buffer for write buffering only.

- Alternate and spare blocks.

- 60 msec operation after power loss to flush about 6MB out of write buffer.  Battery?

http://www.aerodefensetech.com/component/content/article/adt/features/feature-articles/21172

Interfaces

http://www.pcworld.com/article/2921412/ssds-have-a-problem-theyre-getting-too-fast-too-soon.html

- SATA I, II, II  (1.5, 3.0, 6.0 Gbits/sec)

- AHCI (Adaptive Host Controller Interface)  Queue= 32 commands

- MVMe  (Non-Volatile Memory express)  Queue=65K commands using 2 lanes of PCie 2nd generation.

- SATA Express (using PCIe) = 10Gbits/sec which works well with SATA III at 6Gbits/sec.

- DDR4/2400 = 8.8GBytes/sec sequential.

- CPU

  - Dual /quad core   16x PCIe lanes

  - I7-5690x  40x PCIe lanes

- Video and SSD compete for PCIe lanes.

Linux optimization

https://sites.google.com/site/easylinuxtipsproject/ssd

- Format and align to 4KByte boundaries.  Format for EXT4, not BTRFS.

- AHCI BIOS

- Firmware update

- Overprovisioning.  Leave some unallocated and unpartitioned space for alternate blocks.  Recommended is 7% and not to exceed 10GBytes max.

- “noatime” in /etc/fstab.  Eliminate writing last access time.

- Enable TRIM in /etc/rc.local or in cron or just run manually with  “sudo fstrim –v”

- discard ????

- Eliminate swap ???  Depends on if swap is needed.

- replacement scheduler program ???

- If dual boot with Windoze, also need to tweak Windoze.

KODI (formerly XBMC)

- http://kodi.tv

- http://www.tvaddons.ag

Linux install and config

- http://kodi.wiki/view/Installing_XBMC_for_Linux

- http://www.tvaddons.ag/install-addons-kodi/

Bad drive test (Windows)

http://802.11junk.com/jeffl/Failing%20Disk%20Drive/