FLUG  Meeting 03/23/2019

by Jeff Liebermann

5G and Reality

Frequencies (US Only)

- STC - ?

- AT&T – 39GHz

- C Spire – 28GHz

- Sprint – 2.5GHz

- T-Mobile – 600Mhz and 28GHz

- Verizon – 28GHz

FDD vs TDD

- Frequency Division Duplex  (2 frequencies)

- Time Division Duplex (1 frequency)

Benefits of 5G

- Faster – Allegedly does gigabit speeds

- Low latency – Needed for robotics, telemedicine, real time, high frequency trading.

  - Latency=1 way.  RTT=round trip (ping).

  - 12in/nsec, 30cm/nsec,1 mile/msec.

  - 30% slower in fiber.

- Better spectrum efficiency and lower cost per bit.  Pack in more users.

Handset vs bricks

- Handsets only on lower <6GHz frequencies.

- Bricks (hot spots) on all frequencies.

How it works

- Range - power - bandwidth compromise

- Beam steering.  Large base station arrays.  Massive MIMO (128 antennas)

- https://www.mwrf.com/systems/defining-massive-mimo-5g-world

- Densification.  Small cell.

Test cities

- Verizon – Sacramento, Houston, Indianapolis, Smog Angeles.

- https://www.lifewire.com/5g-availability-us-4155914

- https://www.npr.org/2019/03/05/700355632/coming-to-a-city-near-you-5g-fastest-wireless-yet-will-bring-new-services

“Five Myths About 5G, Debunked”

- Video https://www.youtube.com/watch?v=lu9G7ef9uBg

- Race to 5G  (0:14)

- Racing China (0:19)

- It’s all about smarter smartphones  (0:40)

- It’s all about Bandwidth  (1:30)

- 5G will mostly impact the Telecom Industry (2:50)

- 5G will replace 4G-LTE  (3:26)

- The rollout is coming soon  (3:52)

Acronym hell.  (The technology with the most acronyms wins).

- 5G NR (Release 15)

- 5G TF and 5G Home – Proprietary Verizon version of 5G NR.

- 5G E – AT&T marketing at its best.

- https://arstechnica.com/information-technology/2019/03/atts-5g-e-is-actually-slower-than-verizon-and-t-mobile-4g-study-finds/

- 3GPPP  http://www.3gpp.org  http://www.3gpp.org/specifications/67-releases

Opposition, cancer, and such.

- Cancer incidence.  https://seer.cancer.gov/faststats/selections.php?series=cancer

- Environmental Health Trust.  456 YouTube videos on dangers of RF.  https://ehtrust.org

- Scientists for Wired Technology.  http://scientists4wiredtech.com

Misc:

Dish overload.  http://802.11junk.com/jeffl/crud/Dish-overload.jpg