SCCARC Talk on flashlights, LiIon, LiPO, Lumens, Lux, eBay, 18650, etc.

http://www.learnbydestroying.com/jeffl/SCCARC-talk-2017-09-15/SCCARC-talk-2017-09-15.doc

Ver 1.4

- Introduction

  - Lumens = SI unit of luminous flux, equal to the amount of light emitted per second in a unit solid angle of one steradian from a uniform source of one candela.

  - Foot Candles = unit of illumination (now little used) equal to that given by a source of one candela at a distance of one foot (equivalent to one lumen per square foot or 10.764 lux).

  - Lux = SI unit of illuminance, equal to one lumen per square meter.

  - Lux meter, integrating ABS pipe,

  - Lumens are measured with an integrating sphere.
https://www.google.com/search?q=integrating+sphere&tbm=isch

  - Circle of area 1 square meter = 1.13 meter or 44.5 inches diameter.  For a 1 square meter spot on the wall, 1 lux = 1 lumen.  Run demo of measuring lumens with lux meter.

  - White LED is typically 80-100 lumens/watt.  Therefore, if you know the current and the LED forward voltage, you can estimate the light output.  Everything else is loss (lens, reflector, heating, battery sag, etc).  Don’t believe unreal lumens and battery ma-hrs.

  - Yellow/blue balance.  Warm 2700-3200K, Daylight 4000-4500K, Cool 5000-6200 K.
https://cdn.instructables.com/F4P/B8FM/HS0CZZOY/F4PB8FMHS0CZZOY.MEDIUM.jpg
https://i.stack.imgur.com/CT604.png

  - Forums
http://www.candlepowerforums.com
http://budgetlightforum.com

- Batteries

  - Why LiIon?  High capacity, high peak current, high energy density, doesn’t leak, no self-discharge, fairly fast charge, low cost per watt-hr.

  - Types of LiIon and LiPo batteries:
http://batteryuniversity.com/learn/article/types_of_lithium_ion

    - Lithium Cobalt Oxide(LiCoO2)

    - Lithium Manganese Oxide (LiMn2O4)

    - Lithium Nickel Manganese Cobalt Oxide

    - Lithium Iron Phosphate

    - Lithium Titanate

  - http://batteryuniversity.com/learn/article/the_li_polymer_battery_substance_or_hype

  - Size.  18650, 14500, 21700(2170).  Protected vs unprotected.  BMS (battery management system).  Use in electric bicycles and cars.

  - How batteries are rated (ma-hr, 0.2C rating, family of curves).  Try to use ma-hr at operating conditions.

  - CBA II discharge tester.
http://www.learnbydestroying.com/jeffl/battery-tests/18650.jpg

  - http://www.learnbydestroying.com/jeffl/LiPo/

  - Salvage from laptop batteries.

  - Spot welder and nickel wire strips.

  - How to kill LiIon.  Store at full charge.  Store discharged below about 30% SoC.  Don’t overcharge (>4.2v).

  - Counterfeit batteries.
https://batterybro.com/blogs/18650-wholesale-battery-reviews/104619270-can-you-identify-the-fake-lg-hg2-18650-battery

- Flashlights

  - Lens vs reflector.  Front vs side lighting.  CoB (chip on board).  Importance of heat sinking.

  - eBay at it’s worst.  Mostly junk, but at good prices.

  - Switches and control.  Tail vs side switch.

  - Sharp edges, bad threads, misaligned LED,

- Demos

  - Show box of flashlights, batteries, and gadgets.

  - Green lasers.

- Charging

  - Show various chargers.  Specs, limits, quality, safety, junk, price.

  - http://www.lygte-info.dk/info/indexBatteriesAndChargers%20UK.html

  - Junk Charger:
http://lygte-info.dk/review/Review%20Charger%20Bowei%20HC-103W%20UK.html

  - Parallel cell charging.

  - Balance charger.  iMax B6.
https://hobbyking.com/en_us/imax-b6-50w-5a-charger-discharger-1-6-cells-genuine.html

- Misc and trivia

  - XT60 connectors to replace Molex and Anderson Power Pole.
https://www.google.com/search?q=xt60&tbm=isch
https://www.aliexpress.com/store/product/10-Pairs-XT30-XT60-XT90-Yellow-Battery-Connector-Set-4-5mm-Male-Female-Gold-Plated-Banana/1764032_32819798560.html